Chevy 6500XD

- Battery-electric vehicle

- Fast, smooth and quiet

- Maintenance performed by Ryder or trained local dealers

- Vouchers and grants available for vehicle and charging

- Tilt cab for convenience in maintaining

- 22 1/2-inch wheels and tires

- Standard air brakes

- Rear multi-leaf springs; front tapered leaf springs

- 90-degree wide-open front doors

- Wide entry step with cornering lamp

- Regenerative braking

Base Chassis 			Class 6 Low Cab Forward Truck

GVWR 				25,950 lbs

Electric Range 			110 miles

Electric Vehicle Supply Equipment (EVSE) 	J1772 with CCS Combo for DC Fast Charge

Bi-Directional Hardware 		Bi-directional onboard charger- DBC-Series 6.6kW

Availability 			Deliveries beginning in Q4 2018

Battery Capacity 			160 kWh

Battery Manufacturer			LG Chem (prismatic cells)

Charge Time 			20 hours (level 2) 3 - 4 hours (DCFC)

Maximum Speed 			65 mph (electronically limited) 77 mph without governor

Peak Power 			220 kW (295 hp)

Torque Rating 			700 Nm (516 Ft-lbs)

Warranty 				Base Chassis: OEM standard warranty

EV Conversion: Lightning Systems standard warranty (Powertrain 5 year / 60,000 miles)

Options	• DC Fast Charge (50kW) • Analytics (predictive maintenance, route scoring, range analysis, driver behavior, geofencing