RIM PROTECTOR PACKAGING COPY

Front of box-	 -RIM PROTECTOR

-Customize your ride

-Fits 12” to 24” rims

-Made of OEM high quality impact and heat resistant nylon

-All in one design built in cover, no glue or clips are required

-Helps prevent rim damage from curb and poor road conditions

-Limited 5 year warranty

Before

After

Back of Box -

Installation instructions

1/ Using a floor jack or hoist lift the vehicle off the ground so that the tire you are installing RIM PROTECTOR on is not making contact with the ground.

2/ Remove the air completely from the tire.

3/ To allow for an easier installation it is recommended that you use water mixed with dish detergent in an 80/20 mixture. To apply use a spray bottle and spray the mixture between the tire and rim. This will provide lubrication and allow the rim protector to easily go into place.

4/ Using a rubber or nylon hammer strike the rim protectors edge to push the rim protector lip between the tire and rim.

5/ Once the rim protector is completely installed align both ends of the rim protector to determine where to cut to provide the best fit. It is recommended to use cutting pliers to cut the rim protector to the appropriate size.

6/ Inspect to ensure the rim protector is completely and evenly installed around the rim and tire.

7/ Inflate tire to the vehicles recommended pressure.

8/ Check tire pressure a few hours after installing your RIM PROTECTOR to ensure installation was done properly and no air has leaked from the tire.

Rim Protector Canada

14 Cole Court

Brampton, Ontario

Canada

L6P-1A6

Customer service number 1-877-455-4593

www.rimprotectorcanada.ca

www.rimprotectorusa.com