Finishes:

Exterior:

Roof, gutters, downpipes: 	Coloursteel – Sandstone Grey

Window joinery: 		Powdercoated – Sandstone Grey

Cedar weatherboard: 		Stained, Wood-X Avalanche

Plaster: 			Resene Half Cloud

Interior:

Flooring: 			Godfrey Hirst timber laminate flooring – Elegant oak

Carpet: 				Godfrey Hirst – Cockle

Tiles: 				600x600, grey matte finish

Ceiling: 				Resene Alabaster

Trims, doors & windows: 	Resene Alabaster

Walls: 				Resene Sea Fog

Kitchen:

Benchtop: 			Granite stone, white polished

Joinery: 			Timberland, premium oak

Splashback:			Tiled, white, gloss

Appliances:			Fisher & Paykel stainless steel

Tapware:			Methven

Exterior:

Interior:

Kitchen:

Specifications:

Construction:

Foundation:			Engineered concrete waffle slab over gravel raft

Framing:			90mm kiln dried SG8 framing

Intertenancy wall:		Gib Intertenancy Barrier system

Exterior wall insulation:		R2.6 glass wool batts

Ceiling insulation:		R4.0 glass wool batts

Midfloor insulation:		R2.2 glass wool batts

Intertenancy wall insulation	R2.2 glass wool batts

Exterior:

Roof:				Coloursteel tray profile

Fascia & gutter:			Coloursteel

Exterior Cladding:	Vertical cedar weatherboard and Rockcote Integra panel & plaster on ventilated cavity system

Window & door joinery:		Double glazed, powder coated aluminium

Front door:			Aluminium

Garage door:			Sectional door with automatic opener and remotes

Internal:

Ceiling height:			2.4m

Linings:				Painted gib, level 4 finish, aqualine to wet areas

Flooring:			Nylon died carpet with 11mm foam underlay

Internal doors:			Painted, flush finish, hollowcore

Door hardware:			Lever action handles, Satin Chrome

Wardrobe joinery:		Prefinished 16mm white melamine carcass

Landry joinery: 			Prefinished 16mm white melamine carcass and laminate benchtop

Bathroom:

Shower base & walls:		Tiled

Shower door:			Glazed

Vanity:				VCBC wall hung, in timberland oak

WC:				VCBC, back to wall

Tapware:			Methven, chrome

Heated towel rail:		Chrome

Wall heater:			Stainless steel

Mirror:				Wall hung mirror above vanity

Electrical:

Lighting	:			Recessed LED down lights

Heating:	Heat pump high wall mounted, Daikin or Mitsubishi, sized to meet Healthy Homes Regulations

Appliances:	Fisher & Paykel, induction hob, oven, rangehood & dishwasher

Water heater:			Electric 250L cylinder

Power:				Double outlets throughout

Smoke alarms:			To each floor

External works:

Driveway:			Concrete, exposed aggregate and sponge finish

Patio:				Concrete, sponge finish

Soft landscaping:		Plantings as per landscaping plan

Fencing:			To perimeter

Other chattels:			Letterbox, washing line