Accurate & Reliable

for Railway Maintenance

ORPMS

Optical Rail Profile Measurement System

#### The Optical Rail Profile Measurement System was developed for high accuracy measurement of tracks profile. It is also used by personnel dealing with high-speed trains, sensitive to running dynamics.

DMA engineers have expertise across the full range of technical disciplines necessary for developing and continually improving a state-of-the-art track measurement system.

Being a 100% in-house developed system, DMA possess the full set of skills to easily support the product for the entire life cycle.

Availability, maintenance, accuracy over time: all are components of the life cycle cost to evaluate carefully.

## ORPMS Output Measurements

- Right rail profile coordinates

- Left rail profile coordinates

- Gauge calculation

- Curvature

Main Advantages/Benefit for the User

The instrument design is very modular. Different versions exist to meet a wide range of requirements. The parameters to be specified to choose the right version are the following:

ORPMS

Optical Rail Profile Measurement System

DMA Accurate & Reliable for railway Monitoring

ORPMS Technical Features

- Maximum number of points (512 camera lines analyzed; average 250-300 good points, depending on the lenses): at 180 Km/h: 1 profile every 1 m; at 300 Km/h 1 profile every 2 m.

- At reduced number of points (256 camera lines analyzed; average 120-160 good points, depending on the lenses): 300 Km/h 1 profile every 1 m.

- Post processing including linearization and filtering.

- Accuracy up to +/- 0.05 mm

- Average number of points measured on the profile up to 300 depending on measurement speed

- Real time acquisition of the profiles and gauge with an accuracy good enough to compute the equivalent conicity according to the UIC519

- Real time computing of the equivalent conicity according to the UIC519 every 50-100 m, depending upon the train speed

- Compute the equivalent conicity according to the UIC519 for intervals down to 0.5 m, in post processing

- Degree of protection: IP63

- Accuracy (specified as sigma of the error of the estimated position of the point on the profile: +/-0.2 to +/-0.05 mm)

- Measurement range (rail displacement around the center position)

- Temperature range

- Data output (Profile only, Vertical and horizontal alignment only, Both)