|  | FIXPAN |  | Stationary kettles |
| --- | --- | --- | --- |
|  |  |  | Capacity 100, 150, 200, 300, 500 and 550 lt – Electric, Gas and Steam heating PF…100A PF…150A PF…200A PF…300A PF…500A PF…510A |
|  | Constructive Features. • Pans bottoms are constructed in AISI 316 quality stainless steel and are even suitable for boiling food products with a high acidity content. • Exterior cladding, in 1 and AISI 304 quality stainless steel. • Carrying structure, in heavy gauge stainless steel. • Lid is insulated (optional extra ), spring loaded counterbalanced type. • Automatic power operated water filling device for the pan. AUTOCLA VE (pressure lid versions) • Counterbalanced lid fitted with 0,05 bar safety relief valve: an air tight seal is assured by means of screw down clamps and silicone gasket. INDIRECT HEATED VERSIONS • Jacket walls are in AISI 304 quality stainless steel. • Jacket pressure is controlled by a pressure switch, a (0,5 bar) safety valve and pressure gauge. • Automatic water filling into the double jacket ELECTRIC VERSIONS • Heated by means of armoured “Incoloy-800” elements, fitted inside the jacket. • High heating capacities, in accordance with CE DIN 18855 Standards for “Fast cooking equipment (see technical data chart). • Power economiser device: cutsin automatically when selected. | temperature is reached. GAS VERSIONS • Heating by means of high efficiency stainless steel tube burners. Automatic ignition and flame control system without pilot burner. Heating controlled by electronic board, with automatic activation of minimum 2 °C before the SETPOINT, and by the pressure switch. STEA M VERSIONS • Heated by steam, introduced into the jacket from an external mains supply, and fitted with a control valve which provides infinite variable heat input. Panel Board - Functions. - ON/OFF switch. - setting of operating temperature. - setting of cooking temperature. - cooking cycle start/stop. Display/Signal. - failure autotest system. - heating operating visual alarm. - product temperature and set temperature display. - time to end of cycle display. - end of cooking sound alarm. Safety System. - emergency button. - blocking of heating for excess of temperature with manual resettable safety thermostat.. | Applications Stews Goulash Sauces Karamell Onions Jams Stuffings Meat Beans |

| Mod. | Overall dimensions mm |  | Useful capacity | Electric versions |  | Gas versions |  | Steam versions |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PF…A | LxPxH | H1 | LT | V/Hz | KW | V/Hz | Kw | V/Hz | Kg/h |
| 100 | 1025x850x900h | 1650 | 102 | 400/50 3N+PE AC | 16 | 230/50 1N+PE AC | 21 | 230/50 1N+PE AC | 15 |
| 150 |  |  | 139 |  | 18 |  | 21 |  | 23 |
| 200 |  |  | 200 |  | 32 |  | 34.5 |  | 31 |
| 300 |  |  | 300 |  | 36 |  | 48 |  | 46 |
| 500 |  |  | 455 |  | 36 |  | 58 |  | 77 |
| 510 |  |  | 500 |  | 54 |  | 60 |  | 77 |