3

AB

TECHNOLOGIES

399-30 10-POSITION RELAY FUNCTION MODULE

DESCRIPTION:

The 10-position relay function module number 399-30 is a member of the 3AB product line. It comes equipped with ten momentary/latching relay outputs plus one double pole output, eliminating the need for direct wiring connection. Integrating a 3AB wireless remote control to several other products, allows the user to be efficient and help cut down cost in the long run. The 399-30 features:

- Double pole output that can be used to energize a pump motor and valve at the same time.

- Momentary, double pole, and factory programmable latching outputs.

- 10 form C relay outputs.

- Equipped with flange-mount case and the ability to program system output upon consumer request.

- Long-life relays switch DC powered loads up to 10 Amps. Wide operation voltage ranging from 12 to 26 VDC. 399-30 serves as a wireless switching device accessible up to a distance of 1000 feet, when paired with the 3AB Handheld (699-418-X10) transmitter.

- Compatibility with a 3AB transmitters (699-418-X10) that allows the user to create up to 16 million distinct transmitters to receiver relationships, which is conducive in deterring unwanted interactions with other systems.

- The ability to transmit one button at a time.

APPLICATIONS

The 399-30 Module has many uses. This includes but is not limited to:

- General remote control

- Motor control

- Operating remote controlled gates

- Lighting control

- Call systems

- Industrial Automation

CAPABILITY

| Capability | Description |
| --- | --- |
| Momentary | All output could be configured, with capability to stay on as long as a button is pressed. |
| Double Pole | Any one of the outputs can be configured to turn the Double Pole output. For example, it has the capability to power a pump motor and energize valves at the same time. |
| Latch | Standard 399-30 can be configured toggle every time the corresponding buttons are pressed (All outputs can be factory programmed to latch). |

Table 1: 399-30 Module Capabilities descriptions.

INSTALLATION PROCEDURES

Before an RF link between the transmitter and 399-30 receiver is established power must be applied and the following steps must be performed

- Press the LEARN-BUTTON. The LEARN-LED should begin to flash to indicate that the 399-30 is ready to pair with the transmitter.

- Press any one of the authorized transmitter buttons to send its ID.

- Press the LEARN-BUTTON again to exit Learn Mode, or let the 399-30 to time out after 17 seconds. 399-30 is now ready for use.

Note: Upon successful pairing, 399-30 outputs will respond to the transmitter buttons press.

M = momentary

D = output can activate Double Pole output

L = Latch

UL = Unlatch

LR= Long range

SR = Short range

| 3AB 10 Buttons Transmitter | 3AB 399-30 Receiver Outputs |
| --- | --- |
| (699-418-X10) | (399-30) |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| 4 | 4 |
| 5 | 5 |
| 6 | 6 |
| 7 | 7 |
| 8 | 8 |
| 9 | 9 |
| 10 | 10 |

How to Order:

For example: Part number for long-range receiver where output 1-3 momentary double pole, output 5 & 6 are latching is 399-30-1MD2MD3MD5L6L-LR