3

AB

TECHNOLOGIES

399-40 10-POSITION BLUETOOTH RELAY FUNCTION MODULE

DESCRIPTION

399-40 is a member of the 3AB Bluetooth products line. It comes equipped with up to ten momentary/latching relay outputs plus one double pole output eliminating the need for direct wiring connection.

399-40 user interface features an onboard push button switch to place the unit in set-up mode. During the set-up process, 399-40 reads and bonds to Bluetooth devices that are within range. The user uses cell phone’s keypad and enters the unique PIN that only allows the authorized user(s) to control the unit. After the set-up mode, 399-40 will enter bypass mode where it is ready to accept commands.

The 399-40 features:

- Bluetooth enabled.

- Hand-free control system.

- Momentary, double pole, and factory programmable latching outputs.

- 10 Form C Relay Outputs.

- The device comes with flange-mount case.

- Long-life relays switch DC powered loads up to 10 Amps.

- Range 300 feet when paired with Bluetooth enabled cell phone.

- Automatically sync to authorized Bluetooth enabled cell phones when they move into range.

APPLICATIONS

The 399-40 Module has many uses. This includes but is not limited to:

- General remote control.

- Motor control.

- Operating remote controlled gates.

- Lighting control.

- Call systems.

- Industrial Automation.

CAPABILITY

| Capability | Description |
| --- | --- |
| Momentary | All output could be configured, with capability to stay on as long as a button is pressed. |
| Double Pole | Any one of the outputs can be configured to turn the Double Pole output. For example, it has the capability to power a pump motor and energize valves at the same time. |
| Latch | Standard 399-40 can be configured toggle every time the corresponding buttons are pressed (All outputs can be factory programmed to latch). |

Table 1: 399-40 Module Capabilities descriptions.

INSTALLATION PROCEDURES

Before an RF link between the Bluetooth enabled devices and 399-40 receiver is established power must be applied, turn on cell phone’s Bluetooth, and perform the following steps:

- Open downloaded Android APP on your cell phone.

- Press the LEARN-BUTTON located on 399-40 and the LEARN-LED should begin to flash to indicate that the 399-40 is ready to pair with the transmitter.

- Use cell phone’s keypad and enters the unique PIN.

- 399-40 will pair with the transmitter and save its ID. After the pairing process the LEARN-LED will stop flashing. 399-40 is now ready for use.

Note: Upon successful pairing, 399-40 outputs will respond to the transmitter buttons press.

M = momentary

D = output can activate Double Pole output

L = Latch

UL = Unlatch

| Bluetooth Enabled Cell Phone App Buttons | 3AB 399-40 Receiver Outputs |
| --- | --- |
|  | (399-40) |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| 4 | 4 |
| 5 | 5 |
| 6 | 6 |
| 7 | 7 |
| 8 | 8 |
| 9 | 9 |
| 10 | 10 |

How to Order:

For example: Part number for 399-40 receiver where output 1-3 momentary double pole, output 5 & 6 are latching is 399-40-1MD2MD3MD5L6L