3

AB

TECHNOLOGIES

699-418-X10 HANDHELD TRANSMITTER USER GUIDE

DESCRIPTION

The 3AB handheld transmitter (699-418-X10) has wide array of applications, however it is used commonly for general remote control and command applications. Not only has the transmitter been pre-certified for FCC Part 15 (418MHz), but also comes with the ten buttons giving the user the ability to to uniquely customize up to 16 million addresses. Such features makes the transmitter not only user friendly, but is also compatible to the users specific needs.

FEATURES

FCC pre-certified

Utilizes advanced encoder

Long range

Can control up to 10 functions

Developed for indoor and outdoor use

16 million address

APPLICATIONS INCLUDE

| General Remote Control ORDERING INFORMATION |  |
| --- | --- |
| PART # | DESCRIPTION |
| 699-418-X10 | 3AB 10 Buttons Transmitter |

Keyless Entry

Garage/Gate Openers

Call Systems

Home/Industrial Automation

Wire Elimination

OPERATION

In order for the 699-418-X10 handheld transmitter to deliver a more superior, stable, frequency accuracy and security transmission, it must utilize an advance integrated circuit (IC) encoder.

Upon pressing the handheld transmitter button, the internals encoder IC is enabled; allowing the encoder to detect the logic states of the button and transmits an encoded signal to the receiver. The transmitter is configured to transmit one button at a time. This communication between the transmitter and receiver will continue until the button is released and signal is no longer transmitted.

The 399-30 receiver decodes the IC and checks the transmitter’s address bits against the address saved in memory bank. If a match is confirmed, and the receiver recognizes the signal, a corresponding output is activated.  Customers who demand a longer range output up to 1000 feet can couple the 699-418-X10 handheld with 399-30-LR.

SETTING THE TRANSMITTER ADDRESS

699-418-X10 comes with a default address and all button authorized capability. The address is changed by using a paper clip or probe to press the CREATE_ADDR button on the board through the hole in the back of the case.

When the button is depressed, LED will light up in the MODE_IND window, indicating that the address is being created. As long as the button is held down, the address will be randomized, but when the button is released the randomized address is saved and the LED will begin flashing to indicate that the Control Permission may now be set. Press the buttons that 699-418-X10 handheld user will have the authority to access. After a 17 seconds timeout, the process will be sealed and the address and control permissions will be set. 399-30 will need to learn the address before it will accept any transmissions.

BUTTON ASSIGNMENTS

699-418-X10 is available in eight button configurations.

| 3AB 10 Buttons Transmitter | 3AB 399-30 Receiver Outputs |
| --- | --- |
| (699-418-X10) | (399-30) |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| 4 | 4 |
| 5 | 5 |
| 6 | 6 |
| 7 | 7 |
| 8 | 8 |
| 9 | 9 |
| 10 | 10 |

Table 1: 399-30 receiver’s corresponding output to handheld’s transmitter button

BATTERY REPLACEMENT

The 699-418-X10 handheld transmitter utilizes a standard CR2032 lithium button cell.  In normal use, it will provide 1 year of operation. In-orderto replace the battery:

- Remove four screws located at the corners of the enclosure and lift the top cover.

- Once the unit is open, remove the battery by sliding it from beneath the holder. There may be the risk of explosion if the battery is replaced with the wrong type. To avoid exposure, replace it with the same type of battery while observing the polarity shown on the battery holder.