3

AB

TECHNOLOGIES

699-418K KEYFOB TRANSMITTER DATA GUIDE

# DESCRIPTION

The 3AB Keyfob transmitter (699-418K) has a wide array of applications but it is primarily used for general remote control and command applications. Not only has the transmitter been pre-certified for FCC Part 15 (418MHz), but the five button applications system enables the user the ability to uniquely customize up to 16 million addresses. Making the transmitter not only user friendly but also compatible to the users specific needs.

# FEATURES

- FCC pre-certified

- Utilizes advanced encoder

- Long range

- Simple user setup

- 16 million address

- 1 to 5 buttons

# APPLICATIONS INCLUDE

| General Remote Control ORDERING INFORMATION |  |
| --- | --- |
| PART # | DESCRIPTION |
| 699-418K-# | 3AB Keyfob Transmitter |
| # = Number of Buttons, 1 to 5 |  |

- Keyless Entry

- Garage/Gate Openers

- Call Systems

- Home/Industrial Automation

- Wire Elimination

# OPERATION

In order for the 699-418K Keyfob transmitter to deliver a more superior stable frequency, with unmatched accuracy while still being secure, it utilizes an advance encoder (Integrated Circuit) IC. Upon pressing the Keyfob button the internals encoder IC is enabled, which allows the encoder to detect the logic states of the button and transmits an encoded signal to the receiver. This communication between the transmitter and receiver will continue until the button is released and signal is no longer transmitted. The receiver on the other hand, decodes the IC and checks the transmitter’s address bits against the address saved in memory bank. If a match is confirmed and the receiver recognizes the signal a corresponding output is activated.  For our customers who demand a longer range output they can couple the 699-418K Keyfob with a 399-10-LR range up to achieve up to 750 feet signal output.

SETTING THE TRANSMITTER ADDRESS

Upon arrival the 699-418K comes standard with a default address and all button authorized. This will allow the customer to fully access the 16 million addresses and be able to customize it right out of the box.  In order to do so, the user needs to use a paper clip or probe to press the CREATE_ADDR button on the circuit board through the hole in the back of the case. When the button is depressed an LED will light up in the MODE_IND window, indicating that the address is being created.  As long as the button is held down the address will be randomized, but when the button is released the randomized address is saved and the LED will begin flashing to indicate that the Control Permission may now be set. Depending on what number on the Keyfob the customer picks that will the corresponding number that the user will have the authority to access. See Figure 2 for the location of CREATE_ADDR button. After a 17 seconds time out, the process will be sealed and the address and control permissions will be set. 399-10 will need to learn the address before it will accept any transmissions.

BUTTON ASSIGNMENTS

699-418K is available in five button configurations.

| 5 button 3AB Keyfob transmitter (699-418K-5) | 3AB 399-10 Receiver Outputs (399-10) |
| --- | --- |
| S1 | 1 |
| S2 | 2 |
| S3 | 3 |
| S4 | 4 |
| S5 | 5 |

Table 1: The table below shows which receiver output will respond to which Keyfob’s button press.

BATTERY REPLACEMENT

The 699-418K Keyfob transmitter arrives with a standard CR2032 lithium button cell and will only utilize that type of battery. During a normal usage of the transmitter, it will provide up to 1 or 2 years of operation. If ever the need arises to change the battery, the customer can do so by gaining access to the old battery by gently prying apart the two halves of the Keyfob at the seam (fingernails of a coin will do). Once the unit is open, remove the battery by sliding it out from beneath the retainer and replacing it with the new battery whilst being aware of the polarity shown below in Figure 3.There may be the risk of explosion or serious injury if the battery is replaced with a none CR2032 lithium cell, in order to keep that from happening please be sure to replace it with the same type of battery