Outline of Kuna Installation Guide

Front Side: What Is Included:

- Universal Mounting Bracket, 2 Fixture Screws, 2 Junction Box Screws, 2 Decorative Fixture Nuts, 2 wires nuts. Show Pictures

- Length Measurement Tool to adjust the Fixture Screws, emphasize savings in time

- Hook to suspend the Light Fixture while wiring up 3 wires, emphasize savings in a helper

- Kuna Light Fixture

Not Included:

- A Standard A-19 light bulb, recommend 700 Lumens, 60 W equivalent for night views of visitors, LED or CFL bulbs

- Other tools needed: a Philips screw driver, a small plier, silicone sealant, a stepping ladder

Back Side: Installation and WiFi Setup Guide

Installation:

Step 1: Turn off power (at circuit breaker box), remove old light fixture and old mounting bracket, clean the old power wires

Step 2: Install new mounting bracket, use the Length Measurement Tool to adjust the length of the 2 Fixture Screws, use a small plier to tighten the nuts to keep the screws tight

Step 3: Wiring Light Fixture: Use the Hook to temporarily suspend the light fixture to the mounting bracket. Pair Black wire from Fixture to Black wire (power) from Junction Box, insert into a nut and twist till secure. Repeat for the White wire (return). Connect the ground wire (bare) to the Ground Nut on the mounting bracket. Tidy up the wires and push the two nuts inside the junction box. Show picture and use fewer words.

Step 4:  Secure Light Fixture: Remove the Hook. Use Picture to show how to use two hands to hold both sides of the Light Fixture with one finger touching each post for each Fixture Screw. Then align the Fixture Screws into the Post and push through. Secure the two Fixture Nuts by hand.

Step 5: Install a light bulb (750 lumen recommended for high quality night image) and apply silicone sealant between wall and the wall plate at the top, leaving the bottom clear so moisture can drip down.

WiFi Setup:

Step 1: Turn on power, the bottom left light will indicate the status of the system: blinking red (about 30 sec?) means….. blinking blue:: system up and ready for WiFi setup

Step 2: Download App: On the phone, search for KUNA app, then download

Step 3: Setup WiFi: Start KUNA app, Setup, identify the best router in your home for Kuna, OR enter Your Router SSID xxxxxx, Your Router Password xxxxxxx

Step 4: System Setup, Bottom Light should be solid blue after about 1 minute.

Step 5: Check for WiFi signal strength on KUNA App….. If WiFi signal is good, DONE, Enjoy. If WiFi signal is weak, visit www……………………..for help to improve your home WiFi setup.

The Kuna App should be self-explanatory.

Please visit www.xxxxxxx for trouble shooting, maintenance, FAQ and customer support.